SHENZHEN  EONE  ELECTRONICS  CO.,LTD

Specification

for

LCD Module

1602A-1

(V1.2)

1602A-1  LCD Module Specification  Ver1.0	1

| Item | Symbol | Test Condition | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Power Supply for Logic | Vdd | -- | 4.7 | 5.0 | 5.5 | V |
| Operating Voltage for LCD | Vdd-Vo | -- | -- | 5.0 | -- | V |
| Input High voltage | Vih | -- | 2.2 | -- | Vdd | V |
| Input Low voltage | Vil | -- | -0.3 | -- | 0.6 | V |
| Output High voltage | Voh | -Ioh=0.2mA | 2.4 | -- | -- | V |
| Output Low voltage | Vol | Iol=1.2mA | -- | -- | 0.4 | V |
| Power supply current | Idd | Vdd=3.0v | -- | 1.1 | -- | mA |

| Item | Symbol | Min. | Max. | Unit |
| --- | --- | --- | --- | --- |
| Power Supply for logic | Vdd | -0.3 | +7.0 | V |
| Power supply for LCD Drive | Vlcd | Vdd-10.0 | Vdd+0.3 | V |
| Input Voltage | Vi | -0.3 | Vdd+0.3 | V |
| Operating Temperature | Ta | 0 | +50 | ℃ |
| Storage Temperature | Tstg | -10 | +60 | ℃ |

| Item | Description | Unit |
| --- | --- | --- |
| PCB Dimension | 80.0*36.0*1.6 | mm |
| View Dimension | 69.5*14.5 | mm |

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

1.  0  FEATURES

Display Mode: STN,  BLUB

Display Formate: 16 Character x 2 Line

Viewing Direction: 6 O’Clock

Input Data: 4-Bits or 8-Bits interface available

Display Font : 5 x 8 Dots

Power Supply : Single Power Supply (5V±10%)

Driving Scheme : 1/16Duty,1/5Bias

BACKLIGHT（SIDE）：LED（WHITE）

2.0   ABSOLUTE MAXIMUM

3.0ELECTRICAL CHARACTERISTICS

(Ta=25℃;Vdd=3.0V±10%,otherwise specified)

4.0   MECHANICAL PARAMETERS

1602A-1  LCD Module Specification  Ver1.0	2

| No. | Symbol | Level Function |  |
| --- | --- | --- | --- |
| 1 | Vss | -- 0V | Power Supply |
| 2 | Vdd | -- +5V |  |
| 3 | V0 | -- for LCD |  |
| 4 | RS | H/L Register Select: H:Data Input L:Instruction Input |  |
| 5 | R/W | H/L H--Read L--Write |  |
| 6 | E | H,H-L Enable Signal |  |
| 7 8 9 10 | DB0 DB1 DB2 DB3 | H/L H/L H/L Data bus used in 8 bit transfer H/L |  |
| 11 12 13 14 | DB4 DB5 DB6 DB7 | H/L H/L Data bus for both 4 and 8 bit transfer H/L H/L |  |
| 15 | BLA | -- BLACKLIGHT +5V |  |
| 16 | BLK | -- BLACKLIGHT 0V- |  |

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

5. 0  PIN ASSIGNMENT

6.0  BLOCK  DIAGRAM

7.0  POWER  SUPPLY  BLOCK  DIAGRAM

1602A-1  LCD Module Specification  Ver1.0	3

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

8.0  TIMING  CHARACTERISTICS

1602A-1  LCD Module Specification  Ver1.0	4

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

9.0  Display control instruction

The display control instructions control the internal state

of the ST7066U-0A. Instruction is received from MPU to

ST7066U-0A for the display control. The following table shows

various instructions.

1602A-1  LCD Module Specification  Ver1.0	5

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

1602A-1  LCD Module Specification  Ver1.0	6

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

1602A-1  LCD Module Specification  Ver1.0	7

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

1602A-1  LCD Module Specification  Ver1.0	8

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

1602A-1  LCD Module Specification  Ver1.0	9

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

1602A-1  LCD Module Specification  Ver1.0	10

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

1602A-1  LCD Module Specification  Ver1.0	11

SHENZHEN  EONE  ELECTRONICS  CO.,LTD

9.EXTERNAL  DIMENSIONS

PITCH  2.54X(16-1)=38,10±0,2

1	2	3	4	5	6	7	8	9   10  11  12  13  14  15  16

VSSVDD V0  RS R/W  E  DB0DB1DB2DB3DB4DB5DB6DB7BLABLK

深圳市冠晶达电子有限公司

EONE ELECTRONICS CO.,LTD.